MATLAB SIMULATION OF PASSBAND TRANSMISSION WITH BER ANALYSIS UNDER MULTIPATH FADING CHANNELS

Analog and Digital Communication — Experiment 24

| Date of Experiment | 12/10/2026 |
| --- | --- |
| Performed By | Lokesh A |
| Register Number | 192512293 |

1. Aim

To simulate a BPSK transmission system over a multipath fading channel with additive white Gaussian noise (AWGN), detect the received bits, calculate the bit error rate (BER), and compare the transmitted and detected data using MATLAB plots.

2. MATLAB Program

clc; clear; close all; % Parameters N = 1000; bits = randi([0 1],N,1); % BPSK Modulation tx = 2*bits - 1; % Multipath Fading Channel h = [1 0.5 0.3]; fade = filter(h,1,tx); % Add AWGN rx = awgn(fade,10,'measured'); % Detection bits_rx = rx > 0; % BER Calculation [~, BER] = biterr(bits,bits_rx); fprintf('BER = %f\n',BER); % Plots figure; subplot(3,1,1) plot(tx) title('Transmitted BPSK Signal') grid on subplot(3,1,2) plot(rx) title('Received Signal (Multipath + AWGN)') grid on subplot(3,1,3) stem(bits(1:50),'b') hold on stem(bits_rx(1:50),'r') title(['BER = ' num2str(BER)]) legend('Original','Detected') grid on

3. Output

The first plot shows the transmitted BPSK signal, with input bits mapped to −1 and +1. The second plot shows the received signal after the multipath channel and AWGN, where channel effects and noise alter the waveform. The third plot compares the original and detected bits for the first 50 bits and displays the calculated BER.

Figure 1: MATLAB editor, received signal, and BER comparison output.

Figure 2: MATLAB plots for transmitted BPSK, received multipath-plus-AWGN signal, and detected bits.

4. Simulation Parameters

| Parameter | Value |
| --- | --- |
| Number of bits (N) | 1000 |
| Modulation scheme | Binary Phase Shift Keying (BPSK) |
| Multipath channel impulse response | h = [1 0.5 0.3] |
| Channel model | Multipath filtering followed by AWGN |
| Signal-to-noise ratio | 10 dB |
| Detection rule | rx > 0 |
| Performance metric | Bit Error Rate (BER) |

5. Applications

Evaluating digital communication links affected by multipath propagation.

Studying bit detection performance in noisy wireless channels.

Comparing transmitted and detected binary data.

Demonstrating BER as a quantitative measure of communication reliability.

6. Procedure

Generate 1000 random binary bits.

Map each bit to a BPSK symbol using tx = 2*bits − 1.

Pass the transmitted symbols through the specified multipath filter.

Add AWGN at 10 dB SNR.

Detect the bits using a zero-threshold decision rule.

Calculate and display the BER, then plot the transmitted signal, received signal, and first 50 original/detected bits.

7. Result and Discussion

The MATLAB simulation produces the transmitted BPSK waveform, the received waveform after multipath filtering and AWGN, and a comparison of original and detected bits. The BER quantifies the bit-decision errors for the generated data. The numerical BER depends on the randomly generated bit sequence and the noise realization, so it may vary between runs.

8. Conclusion

A BPSK communication link was simulated under multipath channel effects and AWGN using MATLAB. The received samples were detected with a zero-threshold decision rule, and the BER was calculated to evaluate transmission performance. The plots provide a visual comparison of the transmitted signal, the noisy received signal, and the detected bits.

DONE BY

Lokesh A

Register Number: 192512293

B.E. Electronics and Communication Engineering (ECE)