MATLAB SIMULATION OF SINGLE-BIT ERROR DETECTION AND CORRECTION USING SYNDROME VECTORS IN LINEAR BLOCK CODES

Analog and Digital Communication — Experiment 26

| Date of Experiment | 12/10/2026 |
| --- | --- |
| Performed By | Lokesh A |
| Register Number | 192512293 |

1. Aim

To simulate a (7,4) Hamming code in MATLAB, introduce a single-bit error into the encoded codeword, decode the received codeword, and observe the corrected message.

2. MATLAB Program

clc; clear; close all; % (7,4) Hamming Code msg = [1 0 1 1]; % Encode code = encode(msg,7,4,'hamming/binary'); % Introduce Single-Bit Error rx = code; rx(3) = ~rx(3); % Flip one bit % Decode and Correct Error dec = decode(rx,7,4,'hamming/binary'); % Display Results disp('Message Bits:') disp(msg) disp('Encoded Bits:') disp(code) disp('Received Bits (with error):') disp(rx) disp('Corrected Message:') disp(dec) % Plot figure; subplot(2,1,1) stem(code,'filled') title('Encoded Codeword') grid on subplot(2,1,2) stem(rx,'filled') title('Received Codeword with Single-Bit Error') grid on

3. Output

The first plot shows the encoded Hamming codeword. The second plot shows the received codeword after one bit is flipped. The MATLAB Command Window displays the original message, encoded bits, received bits containing the error, and decoded message. The decoder is used to recover the message from the corrupted codeword.

Figure 1: MATLAB command-window output and plotted encoded and received codewords.

Figure 2: Encoded codeword compared with the received codeword containing a single-bit error.

4. Simulation Parameters

| Parameter | Value |
| --- | --- |
| Block code | (7,4) Hamming code |
| Message bits | [1 0 1 1] |
| Codeword length | 7 bits |
| Message length | 4 bits |
| Error introduced | Single-bit error at position 3 |
| Encoding function | encode(msg,7,4,'hamming/binary') |
| Decoding function | decode(rx,7,4,'hamming/binary') |

5. Applications

Error detection and correction in digital communication systems.

Reliable data transmission over noisy communication channels.

Error-control coding in data storage and transmission.

Demonstrating single-bit error correction using Hamming codes.

6. Procedure

Define the four-bit message sequence.

Encode the message using the (7,4) Hamming code.

Flip the bit at position 3 to simulate a single-bit transmission error.

Decode the received codeword using the Hamming decoder.

Display the message, encoded codeword, received codeword, and decoded message.

Plot the encoded and corrupted codewords for comparison.

7. Result and Discussion

The simulation generates a Hamming codeword from the input message and introduces a single-bit error at the third position. The received codeword is then passed to the decoder, and the decoded message is displayed. The plots illustrate the difference between the original encoded codeword and the received codeword after the bit flip.

8. Conclusion

A (7,4) Hamming coding system was simulated in MATLAB to demonstrate single-bit error introduction and correction. The encoded and corrupted codewords were plotted, and the decoder output was displayed to illustrate the role of block codes in improving data reliability.

DONE BY

Lokesh A

Register Number: 192512293

B.E. Electronics and Communication Engineering (ECE)