PERFORMANCE EVALUATION OF 8-QAM AND 16-QAM COMMUNICATION SYSTEMS UNDER AWGN USING BER, CONSTELLATION AND EYE DIAGRAM ANALYSIS IN MATLAB

Analog and Digital Communication — Experiment 23

| Date of Experiment | 12/10/2026 |
| --- | --- |
| Performed By | Lokesh A |
| Register Number | 192512293 |

1. Aim

To simulate a 16-QAM communication system in MATLAB, transmit randomly generated symbols through an AWGN channel, demodulate the received symbols, calculate the bit error rate (BER), and analyse the transmitted and received constellation diagrams and eye diagram.

2. MATLAB Program

clc; clear; close all; % Parameters M = 16; % 16-QAM N = 1000; % Number of symbols data = randi([0 M-1],N,1); % QAM Modulation tx = qammod(data,M,'UnitAveragePower',true); % AWGN Channel rx = awgn(tx,10,'measured'); % 10 dB SNR % Demodulation data_rx = qamdemod(rx,M,'UnitAveragePower',true); % BER Calculation [num, ber] = biterr(data,data_rx); % Display BER fprintf('Bit Error Rate (BER) = %f\n',ber); % Eye Diagram Signal eyeSig = repelem(real(rx),8); % Plots figure; subplot(2,2,1) scatter(real(tx),imag(tx),'filled') title('16-QAM Transmitted Constellation') grid on subplot(2,2,2) scatter(real(rx),imag(rx),'filled') title('Received Constellation (AWGN)') grid on subplot(2,2,[3 4]) eyediagram(eyeSig,16) title('Eye Diagram')

3. Output

The transmitted constellation shows the ideal 16-QAM symbol locations. After the AWGN channel, the received constellation points spread around their ideal positions because of noise. The eye diagram displays the real component of the received signal over overlapping intervals. The program also prints the calculated BER in the MATLAB Command Window.

Figure 1: MATLAB output showing transmitted and received 16-QAM constellations and eye diagram.

Figure 2: Additional MATLAB output supplied for this experiment.

4. Simulation Parameters

| Parameter | Value |
| --- | --- |
| Modulation order (M) | 16-QAM |
| Number of symbols (N) | 1000 |
| Channel model | Additive White Gaussian Noise (AWGN) |
| Signal-to-noise ratio | 10 dB |
| Demodulation | 16-QAM demodulation |
| Performance metric | Bit Error Rate (BER) |
| Eye signal | Real part of received symbols repeated 8 times |

5. Applications

High-data-rate wireless communication systems.

Wi-Fi and broadband communication systems that use QAM-family modulation.

Digital video and data transmission where spectral efficiency is important.

Testing receiver performance by measuring BER and observing constellation distortion under noise.

6. Alignment with Analog and Digital Communication

This experiment demonstrates digital QAM modulation and demodulation, AWGN channel modelling, BER calculation, constellation analysis and eye diagram visualisation. It connects modulation order and noise effects with the reliability of received digital data.

| ADC Concept | Implementation | Observed Output |
| --- | --- | --- |
| Symbol generation | randi([0 M-1],N,1) | Random input symbols |
| 16-QAM modulation | qammod(data,M,'UnitAveragePower',true) | Transmitted constellation |
| Noisy channel | awgn(tx,10,'measured') | Spread received constellation |
| Demodulation | qamdemod(rx,M,'UnitAveragePower',true) | Detected symbols |
| BER calculation | biterr(data,data_rx) | BER printed in Command Window |
| Eye diagram | eyediagram(eyeSig,16) | Eye pattern of real received signal |

7. Justification

Random symbols are used to test the modulation system with a range of possible QAM symbol values.

16-QAM carries multiple bits per symbol and provides higher spectral efficiency than lower-order modulation, at the cost of greater sensitivity to noise.

AWGN introduces channel noise to evaluate how the receiver performs under impaired conditions.

The transmitted and received constellation plots make symbol displacement due to noise visible.

BER gives a numerical measure of detected-data errors, while the eye diagram offers a visual view of the received waveform.

8. Conclusion

A 16-QAM communication system was simulated in MATLAB with an AWGN channel at 10 dB SNR. The received symbols were demodulated, the BER was calculated, and constellation and eye-diagram plots were generated. The results illustrate how noise affects QAM symbol locations and how BER and visual plots can be used together to evaluate digital communication performance.

DONE BY

Lokesh A

Register Number: 192512293

B.E. Electronics and Communication Engineering (ECE)