SIMULATION AND ANALYSIS OF BPSK MODULATION USING CONSTELLATION DIAGRAMS IN MATLAB

Analog and Digital Communication — Experiment 22

| Date of Experiment | 12/10/2026 |
| --- | --- |
| Performed By | Lokesh A |
| Register Number | 192512293 |

1. Aim

To generate random binary data, perform Binary Phase Shift Keying (BPSK) modulation, add Additive White Gaussian Noise (AWGN), and analyse the received symbols using a constellation diagram in MATLAB.

2. MATLAB Program

clc; clear; close all; % Generate Random Bits N = 100; bits = randi([0 1],N,1); % BPSK Modulation bpsk = 2*bits - 1; % 0 -> -1, 1 -> +1 % Add AWGN Noise rx = awgn(bpsk,10,'measured'); % Constellation Diagram figure; subplot(2,1,1) stem(bits) title('Input Binary Data') grid on subplot(2,1,2) scatter(real(rx), imag(rx),'filled') title('BPSK Constellation Diagram') xlabel('In-Phase') ylabel('Quadrature') grid on axis([-2 2 -2 2])

3. Output

The first plot displays the input binary data. The second plot shows the received BPSK symbols in the in-phase and quadrature plane after AWGN is added. Since BPSK uses two ideal symbol states on the in-phase axis, the received points appear around the negative and positive in-phase positions; noise causes the points to spread around those states.

Figure 1: Input binary data and BPSK constellation diagram with AWGN.

4. Simulation Parameters

| Parameter | Value |
| --- | --- |
| Number of bits (N) | 100 |
| Bit generation | randi([0 1],N,1) |
| BPSK mapping | 0 → −1, 1 → +1 |
| Channel model | AWGN |
| Signal-to-noise ratio | 10 dB |
| Constellation axes | In-Phase and Quadrature |

5. Applications

Digital wireless communication links that require robust binary data transmission.

Satellite and deep-space communication links where noise performance is important.

Digital telemetry and data transmission systems.

Teaching and evaluating digital modulation using constellation diagrams.

6. Alignment with Analog and Digital Communication

This experiment covers digital data generation, BPSK symbol mapping, AWGN channel modelling and constellation-diagram analysis. It demonstrates how channel noise changes the received symbol locations and provides a visual way to understand the separation between the two BPSK symbol states.

| ADC Concept | Implementation | Observed Output |
| --- | --- | --- |
| Binary data generation | randi([0 1],N,1) | Input Binary Data |
| BPSK mapping | 2*bits - 1 | Symbols at −1 and +1 ideally |
| AWGN channel | awgn(bpsk,10,'measured') | Noisy received symbols |
| Constellation analysis | scatter(real(rx),imag(rx),'filled') | BPSK Constellation Diagram |

7. Justification

Random binary input provides a data sequence for the modulation process.

BPSK maps each bit to one of two opposite phase states, represented here by −1 and +1.

AWGN models noise that can affect symbol detection at the receiver.

The constellation diagram visualizes received symbols and the spread caused by noise.

The two-state BPSK representation makes it easier to study symbol separation and noise tolerance.

8. Conclusion

BPSK modulation was simulated in MATLAB using a random binary sequence. AWGN was added at an SNR of 10 dB, and the received symbols were displayed using a constellation diagram. The plot shows two groups of received points around the ideal BPSK symbol locations, illustrating the effect of noise on a digital communication signal.

DONE BY

Lokesh A

Register Number: 192512293

B.E. Electronics and Communication Engineering (ECE)