SIMULATION OF CONVOLUTIONAL CODES AND ANALYSIS OF ENCODED OUTPUT BIT SEQUENCES USING MATLAB

Analog and Digital Communication — Experiment 25

| Date of Experiment | 12/10/2026 |
| --- | --- |
| Performed By | Lokesh A |
| Register Number | 192512293 |

1. Aim

To generate a random binary input sequence, encode it using a rate 1/2 convolutional encoder with constraint length 3, and analyse the input and encoded bit sequences using MATLAB.

2. MATLAB Program

clc; clear; close all; % Input Bits data = randi([0 1],20,1); % Convolutional Encoder (Rate 1/2, Constraint Length = 3) trellis = poly2trellis(3,[7 5]); encoded = convenc(data,trellis); % Display disp('Input Bits:'); disp(data'); disp('Encoded Bits:'); disp(encoded'); % Plot figure; subplot(2,1,1) stem(data,'filled') title('Input Bit Sequence') xlabel('Bit Index') ylabel('Bit Value') grid on subplot(2,1,2) stem(encoded,'filled') title('Convolutionally Encoded Bit Sequence') xlabel('Bit Index') ylabel('Bit Value') grid on

3. Output

The first plot displays the 20 randomly generated input bits. The second plot displays the encoded output sequence produced by the convolutional encoder. For a rate 1/2 encoder, the output sequence contains approximately twice as many encoded bits as input bits; the exact displayed sequence depends on the randomly generated input.

Figure 1: MATLAB code and plotted input and convolutionally encoded bit sequences.

Figure 2: Input bit sequence and convolutionally encoded output sequence.

4. Simulation Parameters

| Parameter | Value |
| --- | --- |
| Input sequence length | 20 bits |
| Input bit values | Random binary values: 0 or 1 |
| Encoder type | Convolutional encoder |
| Code rate | 1/2 |
| Constraint length | 3 |
| Generator polynomials | [7 5] in octal notation |
| MATLAB functions | poly2trellis, convenc |

5. Applications

Forward error correction in digital communication systems.

Improving reliability of data transmission over noisy channels.

Encoding data for wireless and mobile communication links.

Studying the relationship between input bits and coded output sequences.

6. Procedure

Generate a 20-bit random binary sequence using randi.

Create a trellis structure using poly2trellis(3,[7 5]).

Encode the input data using convenc.

Display the input bits and encoded bits in the MATLAB Command Window.

Plot both sequences using stem plots and label the axes.

7. Result and Discussion

The input binary sequence was generated and processed by the convolutional encoder. The plotted output demonstrates how the encoder maps the input bits into a longer coded sequence according to the selected rate 1/2 code and generator polynomials. The random input sequence and corresponding encoded values can change on each run.

8. Conclusion

Convolutional encoding was simulated successfully in MATLAB using a rate 1/2 encoder with constraint length 3 and generator polynomials [7 5]. The input and encoded bit sequences were displayed and plotted, illustrating the basic operation of convolutional coding for error-control applications.

DONE BY

Lokesh A

Register Number: 192512293

B.E. Electronics and Communication Engineering (ECE)