SIMULATION OF DSSS COMMUNICATION SYSTEM WITH PN CODE GENERATION AND SIGNAL RECOVERY USING MATLAB

Analog and Digital Communication — Experiment 28

| Date of Experiment | 12/10/2026 |
| --- | --- |
| Performed By | Lokesh A |
| Register Number | 192512293 |

1. Aim

To simulate a Direct Sequence Spread Spectrum (DSSS) communication system in MATLAB using a pseudo-noise (PN) code, spread the input data, and recover the original binary data through despreading.

2. MATLAB Program

clc; clear; close all; % Data Bits data = [1 0 1 1 0]; data = 2*data - 1; % Convert to ±1 % PN Sequence (Length = 5) pn = [1 -1 1 -1 1]; % DSSS Spreading spread = kron(data,pn); % Despreading rec = reshape(spread,length(pn),[]); rec_data = sign(sum(rec)); rec_data = (rec_data+1)/2; % Convert back to binary % Display disp('Original Data:'); disp((data+1)/2); disp('Recovered Data:'); disp(rec_data); % Plots figure; subplot(3,1,1) stem((data+1)/2,'filled') title('Original Data') grid on subplot(3,1,2) stem(spread,'filled') title('DSSS Spread Signal') grid on subplot(3,1,3) stem(rec_data,'filled') title('Recovered Data') grid on

3. Output

The program converts the input bits into bipolar values, spreads each data symbol using the five-chip PN sequence, and despreads the signal by grouping the chips and summing each group. The recovered binary sequence should match the original input data for this ideal, noise-free simulation.

Expected Command Window output:

Original Data:  1  0  1  1  0
Recovered Data:  1  0  1  1  0

4. Simulation Parameters

| Parameter | Value |
| --- | --- |
| Input data bits | 1 0 1 1 0 |
| Bipolar mapping | 0 → −1, 1 → +1 |
| PN sequence | 1 −1 1 −1 1 |
| PN sequence length | 5 chips per data bit |
| Spreading operation | kron(data, pn) |
| Despreading operation | Reshape into 5-chip groups and sum |

5. Procedure

Clear the MATLAB workspace and close all open figures.

Define the binary input data and convert the bits from 0/1 representation to −1/+1 bipolar representation.

Define the five-chip PN sequence.

Spread each data symbol by multiplying it with the PN sequence using the kron function.

Reshape the spread sequence into groups containing five chips and sum each group to despread the signal.

Convert the recovered bipolar values back to binary and display the original and recovered data.

Plot the original data, DSSS spread signal, and recovered data using stem plots.

6. Applications

Direct Sequence Spread Spectrum (DSSS) wireless communication.

Code Division Multiple Access (CDMA) systems.

Improving resistance to narrowband interference.

Spread-spectrum signal synchronization and data recovery.

7. Result and Discussion

The DSSS spreading and despreading process was simulated using MATLAB. Each input bit was represented by five chips from the PN code, producing a spread sequence of 25 chips. Since the simulation does not add channel noise, despreading recovers the original five data bits.

8. Conclusion

The MATLAB simulation demonstrated PN-code-based spreading and recovery in a DSSS communication system. The spread signal was grouped and despread using the same PN sequence, and the recovered data matched the original input sequence.

DONE BY

Lokesh A

Register Number: 192512293

B.E. Electronics and Communication Engineering (ECE)