CALCULATION AND VERIFICATION OF FREQUENCY DEVIATION AND MODULATION INDEX OF FM SIGNALS USING MATLAB SIMULATION

Analog and Digital Communication — Experiment 7

| Date of Experiment | 12/10/2026 |
| --- | --- |
| Performed By | Lokesh A |
| Register Number | 192512293 |

1. Aim

To calculate and verify the frequency deviation and modulation index of a Frequency Modulated (FM) signal using MATLAB, and to plot the message signal and FM signal.

2. MATLAB Program

% Calculation and Verification of Frequency Deviation % and Modulation Index of FM Signal clc; clear; close all; %% Message Signal Parameters Am = 2; % Message amplitude (V) fm = 500; % Message frequency (Hz) %% Carrier Signal Parameters Ac = 1; % Carrier amplitude (V) fc = 10000; % Carrier frequency (Hz) %% FM Parameters kf = 250; % Frequency sensitivity (Hz/V) fs = 100000; % Sampling frequency (Hz) T = 0.01; % Simulation duration (s) %% Time Vector t = 0:1/fs:T; %% Message Signal m = Am*cos(2*pi*fm*t); %% Frequency Deviation delta_f = kf*Am; %% Modulation Index beta = delta_f/fm; %% FM Signal fm_signal = Ac*cos(2*pi*fc*t + beta*sin(2*pi*fm*t)); %% Display Parameters fprintf('Carrier Frequency (fc) = %.0f Hz\n',fc); fprintf('Message Frequency (fm) = %.0f Hz\n',fm); fprintf('Message Amplitude (Am) = %.0f V\n',Am); fprintf('Frequency Sensitivity (kf) = %.0f Hz/V\n',kf); fprintf('\nFrequency Deviation (Delta f) = %.2f Hz\n',delta_f); fprintf('Modulation Index (Beta) = %.2f\n',beta); %% Verification beta_verify = delta_f/fm; fprintf('Verified Modulation Index = %.2f\n',beta_verify); %% Verification Check tolerance = 1e-6; if abs(beta - beta_verify) < tolerance fprintf('\nVerification Successful\n'); else fprintf('\nVerification Failed\n'); end %% Plotting figure; subplot(2,1,1); plot(t,m,'LineWidth',1.5); grid on; title('Message Signal'); xlabel('Time (s)'); ylabel('Amplitude (V)'); subplot(2,1,2); plot(t,fm_signal,'LineWidth',1); grid on; title('FM Signal'); xlabel('Time (s)'); ylabel('Amplitude (V)');

3. Output

The MATLAB output displays the message signal and FM signal. The Command Window reports the carrier frequency, message frequency, message amplitude, frequency sensitivity, calculated frequency deviation, modulation index and verification result.

Figure 1. MATLAB plots of the message signal and FM signal.

Figure 2. MATLAB Command Window showing the calculated parameters and successful verification.

4. Calculated Results

| Parameter | Value |
| --- | --- |
| Message amplitude (Am) | 2 V |
| Message frequency (fm) | 500 Hz |
| Carrier amplitude (Ac) | 1 V |
| Carrier frequency (fc) | 10,000 Hz |
| Frequency sensitivity (kf) | 250 Hz/V |
| Frequency deviation (Δf = kf × Am) | 500 Hz |
| FM modulation index (β = Δf / fm) | 1.00 |
| Verified modulation index | 1.00 |
| Verification status | Successful |

5. Applications

FM radio broadcasting and audio transmission.

Wireless communication systems that encode information through carrier-frequency variation.

FM telemetry and monitoring systems that transmit measurement information.

Communication laboratories for verifying frequency deviation and modulation index.

6. Alignment with Analog and Digital Communication

This experiment directly demonstrates frequency modulation, frequency deviation and FM modulation index, which are fundamental parameters in angle modulation. MATLAB is used to generate the message and FM waveforms and verify the numerical relationship β = Δf / fm.

| ADC Concept | Implementation | Observed Result |
| --- | --- | --- |
| Message signal | Am = 2 V and fm = 500 Hz | Message Signal plot |
| Carrier signal | Ac = 1 V and fc = 10 kHz | Carrier used in FM generation |
| Frequency deviation | Δf = kf × Am | 500 Hz |
| FM modulation index | β = Δf / fm | 1.00 |
| Verification | Compare calculated and verified β values | Verification Successful |
| FM waveform | Phase term includes β sin(2πfmt) | FM Signal plot |

7. Justification

The experiment calculates frequency deviation from frequency sensitivity and message amplitude.

The modulation index is determined using the ratio of frequency deviation to message frequency.

An independent verification calculation checks the modulation-index result.

The message and FM plots visually demonstrate the signals used in the simulation.

The Command Window output provides numerical evidence that the calculated and verified modulation indices agree.

8. Conclusion

The frequency deviation and modulation index of the FM signal were calculated and verified using MATLAB. For the given parameters, the frequency deviation is 500 Hz and the modulation index is 1.00. The verification check was successful, and the message and FM waveforms were plotted.

DONE BY

Lokesh A

Register Number: 192512293

B.E. Electronics and Communication Engineering (ECE)