MATLAB SIMULATION AND EVALUATION OF EYE DIAGRAM PARAMETERS UNDER INTER-SYMBOL INTERFERENCE IN DIGITAL COMMUNICATION SYSTEMS

Analog and Digital Communication — Experiment 18

| Date of Experiment | 12/10/2026 |
| --- | --- |
| Performed By | Lokesh A |
| Register Number | 192512293 |

1. Aim

To simulate a digital NRZ communication signal in MATLAB, introduce inter-symbol interference (ISI) using a channel filter, and evaluate the received waveform using an eye diagram.

2. MATLAB Program

clc; clear; close all; %% Parameters N = 100; % Number of bits sps = 20; % Samples per symbol %% Generate Random Binary Data data = randi([0 1],1,N); %% NRZ Encoding tx = repelem(data,sps); %% Time Axis t = (0:length(tx)-1)/sps; %% Introduce Inter-Symbol Interference (ISI) % Simple Low-Pass FIR Filter (Channel) h = [0.2 0.6 1 0.6 0.2]; % Received Signal with ISI rx = conv(tx,h,'same'); %% Plot Results figure; subplot(4,1,1) stairs(data,'LineWidth',1.5) grid on title('Original Binary Data') xlabel('Bit Number') ylabel('Bit') ylim([-0.2 1.2]) subplot(4,1,2) plot(t,tx,'LineWidth',1.5) grid on title('Transmitted NRZ Signal') xlabel('Time') ylabel('Amplitude') ylim([-0.5 1.5]) subplot(4,1,3) plot(t,rx,'r','LineWidth',1.2) grid on title('Received Signal with Inter-Symbol Interference (ISI)') xlabel('Time') ylabel('Amplitude') subplot(4,1,4) eyediagram(rx,2*sps) title('Eye Diagram under ISI')

3. Output

The simulation displays the original binary data, transmitted NRZ waveform, received waveform after the channel filter, and the eye diagram under ISI. The channel filter spreads each symbol's response over neighbouring time intervals, which can cause adjacent symbols to interfere with one another.

4. Simulation Parameters

| Parameter | Value |
| --- | --- |
| Number of bits (N) | 100 |
| Samples per symbol (sps) | 20 |
| Line coding | NRZ (0 → 0, 1 → 1) |
| Channel model | FIR filter with coefficients [0.2 0.6 1 0.6 0.2] |
| Received signal | rx = conv(tx,h,'same') |
| Eye diagram window | 2 × sps = 40 samples |

5. Applications

Studying inter-symbol interference in digital baseband communication channels.

Evaluating eye-pattern closure and symbol overlap caused by channel distortion.

Demonstrating the effects of bandwidth-limited channels on digital signals.

Supporting communication receiver design and channel equalization studies.

6. Alignment with Analog and Digital Communication

This experiment demonstrates digital NRZ signalling, channel response, inter-symbol interference and eye diagram analysis. It connects the transmitted symbol sequence with the distortion introduced by a channel and shows how that distortion can affect reliable symbol detection.

| ADC Concept | Implementation | Observed Output |
| --- | --- | --- |
| Binary data | randi([0 1],1,N) | Original Binary Data |
| NRZ encoding | repelem(data,sps) | Transmitted NRZ Signal |
| Channel response | h = [0.2 0.6 1 0.6 0.2] | Signal spread by channel filtering |
| ISI signal | rx = conv(tx,h,'same') | Received Signal with ISI |
| Eye diagram | eyediagram(rx,2*sps) | Eye Diagram under ISI |

7. Justification

Random binary data represents the information sequence sent by the digital transmitter.

NRZ encoding converts each bit into a constant-level waveform over its symbol period.

The FIR channel filter spreads the signal response in time, modelling a simple source of ISI.

Convolution models the effect of the channel response on the transmitted signal.

The eye diagram provides a visual tool for examining signal overlap and potential symbol-decision difficulty.

8. Conclusion

A digital NRZ signal was generated in MATLAB and passed through a simple FIR channel model to introduce inter-symbol interference. The original data, transmitted waveform, received waveform and eye diagram were plotted. The simulation demonstrates how channel-induced pulse spreading can cause adjacent symbols to overlap and affect digital communication performance.

DONE BY

Lokesh A

Register Number: 192512293

B.E. Electronics and Communication Engineering (ECE)