PERFORMANCE ANALYSIS OF FM SYSTEM UNDER GAUSSIAN WHITE NOISE USING MATLAB

Analog and Digital Communication — Experiment 10

| Date of Experiment | 12/10/2026 |
| --- | --- |
| Performed By | Lokesh A |
| Register Number | 192512293 |

1. Aim

To generate a frequency-modulated (FM) signal in MATLAB, introduce additive Gaussian white noise, and observe the effect of noise on the received FM waveform.

2. MATLAB Program

clc; clear; close all; % Parameters fs = 100000; t = 0:1/fs:0.02; fm = 500; % Message frequency fc = 5000; % Carrier frequency kf = 100; % Frequency sensitivity % Message Signal m = cos(2*pi*fm*t); c = cos(2*pi*fc*t); % FM Modulation fm_sig = cos(2*pi*fc*t + 2*pi*kf*cumsum(m)/fs); % Add Gaussian White Noise noisy = awgn(fm_sig,10,'measured'); % 10 dB SNR % Display figure; subplot(4,1,1) plot(t,m) title('Message Signal') grid on subplot(4,1,2) plot(t,m) title('Carrier Signal') grid on subplot(4,1,3) plot(t,fm_sig) title('FM Signal') grid on subplot(4,1,4) plot(t,noisy) title('FM Signal with Gaussian White Noise') grid on

3. Output

The plots display the message signal, the signal labelled as the carrier in the supplied program, the FM signal, and the FM signal after adding Gaussian white noise. The noisy waveform shows random amplitude variations caused by the simulated noise.

Figure 1. MATLAB output showing the message, carrier-labelled waveform, FM signal and noisy FM signal.

Figure 2. MATLAB editor and graphical output for FM performance under Gaussian white noise.

4. Simulation Parameters

| Parameter | Value |
| --- | --- |
| Sampling frequency (fs) | 100,000 Hz |
| Message frequency (fm) | 500 Hz |
| Carrier frequency (fc) | 5,000 Hz |
| Frequency sensitivity (kf) | 100 |
| Simulation duration | 0.02 s |
| Noise model | Additive Gaussian White Noise |
| Signal-to-noise ratio (SNR) | 10 dB |

5. Applications

Evaluating FM signal behaviour in noisy communication channels.

Studying noise effects in FM receivers and communication-system simulations.

Testing and demonstrating the role of signal-to-noise ratio in received-signal quality.

Educational analysis of angle modulation and channel noise in Analog and Digital Communication.

6. Alignment with Analog and Digital Communication

This experiment demonstrates frequency modulation and the effect of additive white Gaussian noise (AWGN) on a modulated signal. MATLAB is used to construct the FM waveform and compare it with the noisy waveform, linking modulation theory with channel impairment.

| ADC Concept | Implementation | Observed Output |
| --- | --- | --- |
| Message signal | m = cos(2*pi*fm*t) | Message Signal plot |
| Carrier expression | c = cos(2*pi*fc*t) | Program defines carrier; plotting line currently uses m |
| FM generation | fm_sig = cos(2*pi*fc*t + 2*pi*kf*cumsum(m)/fs) | FM Signal plot |
| Noisy channel | awgn(fm_sig,10,'measured') | FM Signal with Gaussian White Noise plot |
| SNR | 10 dB | Noise level set by AWGN function |

7. Justification

The message signal provides the low-frequency information used to vary the instantaneous phase of the carrier in FM.

The FM waveform demonstrates angle modulation with a constant nominal amplitude.

AWGN is added at a specified 10 dB SNR to model random channel noise.

Comparing the FM and noisy FM plots visually demonstrates the effect of noise on the received waveform.

The supplied code plots m in the second subplot despite labelling it 'Carrier Signal'; the document preserves the given program rather than silently changing it.

8. Conclusion

The FM signal was generated using MATLAB and additive Gaussian white noise was introduced at an SNR of 10 dB. The message, carrier-labelled waveform, FM waveform and noisy FM waveform were plotted. The comparison illustrates the influence of channel noise on an FM signal.

DONE BY

Lokesh A

Register Number: 192512293

B.E. Electronics and Communication Engineering (ECE)