PERFORMANCE EVALUATION OF DIGITAL COMMUNICATION SYSTEMS USING EYE DIAGRAM ANALYSIS UNDER ADDITIVE WHITE GAUSSIAN NOISE (AWGN) USING MATLAB

Analog and Digital Communication — Experiment 17

| Date of Experiment | 12/10/2026 |
| --- | --- |
| Performed By | Lokesh A |
| Register Number | 192512293 |

1. Aim

To generate a random binary sequence, encode it using NRZ signalling, pass the transmitted signal through an AWGN channel, and evaluate the received signal using an eye diagram in MATLAB.

2. MATLAB Program

clc; clear; close all; %% Parameters N = 100; % Number of bits sps = 20; % Samples per symbol SNR = 10; % Signal-to-Noise Ratio (dB) %% Generate Random Binary Data data = randi([0 1],1,N); %% NRZ Encoding (0 -> 0, 1 -> 1) tx_signal = repelem(data,sps); %% Pass Signal through AWGN Channel rx_signal = awgn(tx_signal,SNR,'measured'); %% Time Axis t = (0:length(tx_signal)-1)/sps; %% Plot Results figure; % Original Binary Data subplot(4,1,1) stairs(data,'LineWidth',1.5) grid on title('Original Binary Data') xlabel('Bit Number') ylabel('Bit') ylim([-0.2 1.2]) % Transmitted NRZ Signal subplot(4,1,2) plot(t,tx_signal,'LineWidth',1.5) grid on title('Transmitted NRZ Signal') xlabel('Time') ylabel('Amplitude') ylim([-0.5 1.5]) % Received Signal after AWGN subplot(4,1,3) plot(t,rx_signal,'r') grid on title(['Received Signal with AWGN (SNR = ',num2str(SNR),' dB)']) xlabel('Time') ylabel('Amplitude') % Eye Diagram subplot(4,1,4) eyediagram(rx_signal,2*sps) title('Eye Diagram of Received Signal')

3. Output

The simulation displays the original binary data, transmitted NRZ signal, received signal after AWGN is added, and the eye diagram of the received signal. The eye diagram visualizes the combined signal transitions over multiple symbol intervals and helps assess noise effects and the clarity of symbol decisions.

4. Simulation Parameters

| Parameter | Value |
| --- | --- |
| Number of bits (N) | 100 |
| Samples per symbol (sps) | 20 |
| Signal-to-noise ratio (SNR) | 10 dB |
| Line coding | NRZ (0 → 0, 1 → 1) |
| Channel model | Additive White Gaussian Noise (AWGN) |
| Eye diagram window | 2 × sps = 40 samples |

5. Applications

Evaluating digital receiver signal quality in noisy communication channels.

Observing timing and noise effects in binary baseband transmission.

Teaching and testing line coding, channel noise and signal detection concepts.

Comparing communication-system performance under different SNR values.

6. Alignment with Analog and Digital Communication

This experiment demonstrates binary data generation, NRZ line coding, AWGN channel modelling and eye diagram analysis. These concepts are used to study how noise affects the received waveform and how clearly digital symbols can be distinguished.

| ADC Concept | Implementation | Observed Output |
| --- | --- | --- |
| Binary data | randi([0 1],1,N) | Original Binary Data |
| NRZ encoding | repelem(data,sps) | Transmitted NRZ Signal |
| Noisy channel | awgn(tx_signal,SNR,'measured') | Received Signal with AWGN |
| Eye diagram | eyediagram(rx_signal,2*sps) | Received-signal eye pattern |
| SNR setting | SNR = 10 dB | Noise level controlled by SNR |

7. Justification

Random binary data provides a digital information sequence for transmission.

NRZ encoding represents each bit by a constant level across its symbol interval.

AWGN introduces controlled noise to simulate channel impairment.

The received waveform shows the combined effect of transmission and noise.

The eye diagram helps visually assess signal separation, timing variation and noise margin.

8. Conclusion

A binary NRZ signal was generated in MATLAB and passed through an AWGN channel at an SNR of 10 dB. The transmitted and received waveforms were plotted, and an eye diagram was produced to visualize the received signal over repeated symbol intervals. This simulation demonstrates how eye diagrams can be used to assess digital communication signal quality in the presence of noise.

DONE BY

Lokesh A

Register Number: 192512293

B.E. Electronics and Communication Engineering (ECE)