POWER SPECTRAL DENSITY (PSD) ANALYSIS OF PHASE MODULATED (PM) SIGNALS USING MATLAB

Analog and Digital Communication — Experiment 8

| Date of Experiment | 12/10/2026 |
| --- | --- |
| Performed By | Lokesh A |
| Register Number | 192512293 |

1. Aim

To generate a Phase Modulated (PM) signal using MATLAB, obtain its frequency spectrum and power spectral density (PSD), and calculate the phase deviation and approximate bandwidth.

2. MATLAB Program

clc; clear; close all; % Parameters Am = 1; % Message Amplitude (V) fm = 100; % Message Frequency (Hz) Ac = 5; % Carrier Amplitude (V) fc = 1000; % Carrier Frequency (Hz) fs = 20*fc; % Sampling Frequency (Hz) kp = pi/2; % Phase Sensitivity (rad/V) % Time Vector t = 0:1/fs:4/fm; % Message Signal m = Am*sin(2*pi*fm*t); % PM Signal pm = Ac*cos(2*pi*fc*t + kp*m); % Phase Deviation beta = kp*Am; % FFT N = length(pm); f = linspace(-fs/2,fs/2,N); PM_FFT = abs(fftshift(fft(pm)))/N; % Power Spectral Density PSD = (abs(fftshift(fft(pm))).^2)/(N*fs); % Approximate Bandwidth BW = 2*(beta+1)*fm; fprintf('Phase Deviation = %.2f radians\n',beta); fprintf('Estimated Bandwidth = %.2f Hz\n',BW); % Plotting figure('Name','PSD Analysis of PM Signal','NumberTitle','off'); subplot(2,2,1); plot(t,m); title('Message Signal'); xlabel('Time (s)'); ylabel('Amplitude'); grid on; subplot(2,2,2); plot(t,pm); title('Phase Modulated Signal'); xlabel('Time (s)'); ylabel('Amplitude'); grid on; subplot(2,2,3); plot(f,PM_FFT); title('Frequency Spectrum of PM Signal'); xlabel('Frequency (Hz)'); ylabel('Magnitude'); grid on; subplot(2,2,4); plot(f,PSD); title('Power Spectral Density'); xlabel('Frequency (Hz)'); ylabel('PSD'); grid on;

3. Output

The MATLAB simulation displays the message signal, phase-modulated signal, frequency spectrum and power spectral density. The Command Window reports the phase deviation and estimated bandwidth.

Figure 1. MATLAB plots showing the message signal, phase-modulated signal, frequency spectrum and PSD.

Figure 2. MATLAB Command Window and graphical output for PSD analysis.

4. Calculated Results

| Parameter | Value |
| --- | --- |
| Message amplitude (Am) | 1 V |
| Message frequency (fm) | 100 Hz |
| Carrier amplitude (Ac) | 5 V |
| Carrier frequency (fc) | 1000 Hz |
| Sampling frequency (fs) | 20,000 Hz |
| Phase sensitivity (kp) | π/2 rad/V |
| Phase deviation (β = kp × Am) | 1.57 radians (approximately) |
| Estimated bandwidth (BW = 2(β + 1)fm) | 514.16 Hz (as displayed in the supplied output) |

5. Applications

Analysis and design of phase-modulated communication systems.

Study of spectral characteristics in angle-modulation systems.

Digital phase modulation systems, including Phase Shift Keying (PSK).

Communication engineering laboratories for comparing signal spectra and power distribution.

6. Alignment with Analog and Digital Communication

This experiment demonstrates phase modulation and spectral analysis. The message signal changes the carrier phase according to the phase sensitivity, and the FFT-based plots show the frequency-domain characteristics and estimated power spectral density of the PM signal.

| ADC Concept | Implementation | Observed Output |
| --- | --- | --- |
| Message signal | 1 V, 100 Hz sinusoid | Message Signal plot |
| Phase modulation | pm = Ac*cos(2*pi*fc*t + kp*m) | Phase Modulated Signal plot |
| Phase deviation | β = kp × Am | Approximately 1.57 radians |
| Frequency spectrum | FFT and fftshift | Frequency Spectrum plot |
| Power spectral density | PSD = ¦FFT¦²/(N × fs) | Power Spectral Density plot |
| Approximate bandwidth | BW = 2(β + 1)fm | 514.16 Hz as displayed in the supplied output |

7. Justification

The message and phase-modulated plots show the time-domain behavior of the PM system.

The frequency spectrum shows how the signal's frequency components are distributed around the carrier.

The PSD plot provides a view of how signal power is distributed across frequency.

The phase deviation is calculated directly from phase sensitivity and message amplitude.

The estimated bandwidth is reported using the formula implemented in the supplied MATLAB program.

8. Conclusion

A phase-modulated signal was generated and analyzed using MATLAB. The time-domain waveforms, frequency spectrum and power spectral density were plotted. The phase deviation was calculated as approximately 1.57 radians, and the estimated bandwidth was reported as 514.16 Hz in the supplied output.

DONE BY

Lokesh A

Register Number: 192512293

B.E. Electronics and Communication Engineering (ECE)