SIMULATION AND ANALYSIS OF UNIFORM AND NON-UNIFORM QUANTIZATION USING MATLAB

Analog and Digital Communication — Experiment 16

| Date of Experiment | 12/10/2026 |
| --- | --- |
| Performed By | Lokesh A |
| Register Number | 192512293 |

1. Aim

To simulate and compare uniform quantization and non-uniform quantization using the μ-law companding method in MATLAB.

2. MATLAB Program

clc; clear; close all; % Input Signal t = 0:0.001:1; x = sin(2*pi*5*t); % Uniform Quantization (8 Levels) L = 8; x_uni = round((x+1)*(L-1)/2); x_uni = (x_uni*2/(L-1))-1; % Non-Uniform Quantization (μ-law) mu = 255; x_comp = sign(x).*log(1+mu*abs(x))/log(1+mu); x_non = round((x_comp+1)*(L-1)/2); x_non = (x_non*2/(L-1))-1; % Plot Results figure; subplot(3,1,1) plot(t,x) title('Original Signal') grid on subplot(3,1,2) stairs(t,x_uni) title('Uniform Quantization') grid on subplot(3,1,3) stairs(t,x_non) title('Non-Uniform Quantization (\mu-law)') grid on

3. Output

The output contains the original sinusoidal signal, its uniformly quantized version with 8 levels, and the non-uniformly quantized version using μ-law companding. The stair-step plots show the discrete quantization levels used to represent the signal amplitude.

Figure 1. MATLAB plots of the original signal, uniform quantization and non-uniform μ-law quantization.

Figure 2. MATLAB editor and output for the quantization simulation.

4. Simulation Parameters

| Parameter | Value |
| --- | --- |
| Input signal | x = sin(2π × 5t) |
| Time vector | 0 to 1 s in steps of 0.001 s |
| Quantization levels (L) | 8 |
| Quantization method 1 | Uniform quantization |
| Quantization method 2 | Non-uniform quantization using μ-law |
| μ-law parameter (μ) | 255 |

5. Applications

Digital telephony and speech coding, where μ-law companding is commonly used.

Analog-to-digital conversion of signals with a wide range of amplitudes.

Audio and speech processing systems requiring improved representation of low-amplitude signals.

Comparative study of quantization error and quantizer characteristics in communication systems.

6. Alignment with Analog and Digital Communication

This experiment covers quantization, a key step in converting analog signals into digital representations. It compares uniform quantization with μ-law non-uniform quantization, illustrating how companding changes the distribution of quantization levels.

| ADC Concept | Implementation | Observed Output |
| --- | --- | --- |
| Original signal | x = sin(2*pi*5*t) | Original Signal plot |
| Uniform quantization | L = 8; round and rescale | Uniform Quantization stair-step plot |
| μ-law companding | sign(x).*log(1+μ¦x¦)/log(1+μ) | Companded signal representation |
| Non-uniform quantization | Quantize the μ-law companded signal | Non-Uniform Quantization (μ-law) plot |
| Comparison | Compare the two quantized outputs | Different quantization step distributions |

7. Justification

Uniform quantization uses evenly spaced representation levels over the normalized input range.

The μ-law operation compresses the signal dynamically before quantization, changing how the levels are allocated.

The staircase plots make the quantization process visible and support comparison of the two methods.

Using the same input signal and eight output levels provides a direct visual comparison of the quantization methods.

8. Conclusion

Uniform and non-uniform quantization were simulated in MATLAB using an 8-level quantizer. The μ-law method was applied to the input sinusoid before quantization, and the resulting stair-step waveform was compared with uniform quantization. The plots demonstrate the different amplitude-level representations produced by the two methods.

DONE BY

Lokesh A

Register Number: 192512293

B.E. Electronics and Communication Engineering (ECE)